Steps with Breadboard

Parts Checklist

- Microbit

- Breadboard

- Big Battery Pack

- Small Battery Pack

- 4 Wires Alligator-Pin

- Motor

- 2 LEDs

- Breadboard

- 4 Pin/Hole Wires

  - 4 for LEDs to be attached to breadboard

  - 1 for Motor attached to battery pack

- 2 Pin/Pin Wires

  - 2 for Motor attached to breadboard

- LEDs

  - Take both LEDs and 4 Pin/Hole wires and attach all F ends to LED pins

    - Make note of which wires are the longer/shorter legs of LED

      - Ex: Yellow is the short leg, Green is the long leg

      - What does this mean?

        - Short leg (-) is goes to GND, Longer leg (+) goes to 3V

    - Take the wire attached to the shorter leg, and plug it into the negative column on the breadboard, in any row - do this for both LEDs

  - Connect LED 1

    - Take Alligator Clip 1, and clamp it to Pin 1 on Microbit

    - With other end of wire, plug it in to Row 1 on the breadboard

    - Take LED 1, and plug the remaining wire (which should be connected to the longer leg) and plug it into Row 1

  - Connect LED 2

    - Take Alligator Clip 2, and clamp it to Pin 2 on Microbit

    - With other end of wire, plug it in to Row 2 on the breadboard

    - Take LED 2, and plug its remaining wire (which should be connected to the longer leg) and plug it into Row 2

- Motor

  - Take 2 Pin/Pin wires, and 1 Pin/Hole wire and attach to the grouped wires coming from the motor

    - Keep track of which wire corresponds to each color of the grouped wires

    - The Pin/Hole wire should correspond to the RED wire coming from the motor

  - With the wire corresponding to the BLACK wire coming from the motor, plug it into the negative column on the breadboard

  - With the wire corresponding to the YELLOW wire coming from the motor, plug it into Row 3 on the breadboard

  - Take Alligator Clip 3, and clamp it on to Pin 0 on Microbit

  - With other end of the wire, plug it in to Row 3 on the breadboard

- GND

  - Take Alligator Clip 4, and clamp it to GND on Microbit

  - With other end of wire, plug in to negative column on breadboard in any row

- Batteries

  - Plug small battery back into microbit

  - Take the Pin/Hole wire attached to the red wire coming from the motor, and attach the F end to the Red wire coming from the big battery pack

  - Take the back wire coming from the big battery pack and plug it into the negative column on the breadboard in any row

  - Turn on the big battery pack

Steps without Full Breadboard

Parts Checklist

- Microbit

- Negative/Positive Column of the Breadboard

- Big Battery Pack

- Small Battery Pack

- 4 Wires Alligator-Pin

- Motor

- 2 LEDs

- Breadboard

- 5 Pin/Hole Wires

  - 4 for LEDs to be attached to breadboard

  - 1 for Motor attached to battery pack

- 1 Pin/Pin Wires

  - 1 for Motor attached to breadboard neg column

- 3+ Hole/Hole Wires

  - 2 for LED-MB

  - 1 for Motor-MB

- LEDs

  - Take both LEDs and 4 Pin/Hole wires and attach all F ends to LED pins

    - Make note of which wires are the longer/shorter legs of LED

      - Ex: Yellow is the short leg, Green is the long leg

      - What does this mean?

        - Short leg (-) is goes to GND, Longer leg (+) goes to 3V

    - Take the wire attached to the shorter leg, and plug it into the negative column on the breadboard, in any row - do this for both LEDs

  - Connect LED 1

    - Take Alligator Clip 1, and clamp it to Pin 1 on Microbit

    - With other end of wire, plug it in to Hole/Hole wire

    - Take LED 1, and plug the remaining wire (which should be connected to the longer leg) and plug it into the other end of the Hole/Hole wire

  - Connect LED 2

    - Take Alligator Clip 2, and clamp it to Pin 2 on Microbit

    - With other end of wire, plug it in to Hole/Hole wire

    - Take LED 2, and plug its remaining wire (which should be connected to the longer leg) and plug it into the other end of the Hole/Hole wire

- Motor

  - Take 1 Pin/Pin wire, 1 Alligator Clip-M wire, and 1 Pin/Hole wire and attach to the grouped wires coming from the motor

    - Keep track of which wire corresponds to each color of the grouped wires

    - The Pin/Hole wire should correspond to the RED wire coming from the motor

    - The Pin/Pin wire should correspond with the BLACK wire

    - The Alligator Pin should correspond with the YELLOW wire

  - With the wire corresponding to the BLACK wire coming from the motor, plug it into the negative column on the breadboard

  - With the wire corresponding to the YELLOW wire coming from the motor, clamp it on to Pin 0 of the microbit

  - Take Alligator Clip 3, and clamp it on to Pin 0 on Microbit

- GND

  - Take Alligator Clip 4, and clamp it to GND on Microbit

  - With other end of wire, plug in to negative column on breadboard in any row

- Batteries

  - Plug small battery back into microbit

  - Take the Pin/Hole wire attached to the red wire coming from the motor, and attach the F end to the Red wire coming from the big battery pack

  - Take the back wire coming from the big battery pack and plug it into the negative column on the breadboard in any row

  - Turn on the big battery pack

Steps with Ceibal’s Board

Parts Checklist

- Microbit

- Ceibal Board  -

- Big Battery Pack

- Small Battery Pack

- 4 Wires Alligator-Pin

- Motor

- 2 LEDs

- Breadboard

- 4 Pin/Hole Wires

  - 4 for LEDs to be attached to breadboard

  - 1 for Motor attached to battery pack

- 2 Pin/Pin Wires

  - 2 for Motor attached to breadboard

- LEDs

  - Take both LEDs and 4 Pin/Hole wires and attach all F ends to LED pins

    - Make note of which wires are the longer/shorter legs of LED

      - Ex: Yellow is the short leg, Green is the long leg

      - What does this mean?

        - Short leg (-) is goes to GND, Longer leg (+) goes to 3V

    - Take the wire attached to the shorter leg, and plug it into row 10 of the board (just has to be same as wherever MB’s ground is)  - do this for both LEDs

  - Connect LED 1

    - Take Alligator Clip 1, and clamp it to Pin 1 on Microbit

    - With other end of wire, plug it in to Row 1 on the breadboard

    - Take LED 1, and plug the remaining wire (which should be connected to the longer leg) and plug it into Row 1

  - Connect LED 2

    - Take Alligator Clip 2, and clamp it to Pin 2 on Microbit

    - With other end of wire, plug it in to Row 2 on the breadboard

    - Take LED 2, and plug its remaining wire (which should be connected to the longer leg) and plug it into Row 2

- Motor

  - Take 2 Pin/Pin wires, and 1 Pin/Hole wire and attach to the grouped wires coming from the motor

    - Keep track of which wire corresponds to each color of the grouped wires

    - The Pin/Hole wire should correspond to the RED wire coming from the motor

  - With the wire corresponding to the BLACK wire coming from the motor, plug it into row 10 of the board (just has to be same as wherever MB’s ground is)

  - With the wire corresponding to the YELLOW wire coming from the motor, plug it into Row 3 on the breadboard

  - Take Alligator Clip 3, and clamp it on to Pin 0 on Microbit

  - With other end of the wire, plug it in to Row 3 on the breadboard

- GND

  - Take Alligator Clip 4, and clamp it to GND on Microbit

  - With other end of wire, plug in to row 10 of the board

- Batteries

  - Plug small battery back into microbit

  - Take the Pin/Hole wire attached to the red wire coming from the motor, and attach the F end to the Red wire coming from the big battery pack

  - Take the back wire coming from the big battery pack and plug it into the negative column on the breadboard in any row

  - Turn on the big battery pack

Converting to ScratchJr Project

Steps with Breadboard

Parts Checklist

- Microbit

- Breadboard

- Big Battery Pack

- Small Battery Pack

- 4 Wires Alligator-Pin

- Motor

- 2 LEDs

- 4 Pin/Hole Wires

  - 4 for LEDs to be attached to breadboard

  - 1 for Motor attached to battery pack

- 2 Pin/Pin Wires

  - 2 for Motor attached to breadboard

- LEDs

  - Take both LEDs and 4 Pin/Hole wires and attach all F ends to LED pins

    - Make note of which wires are the longer/shorter legs of LED

      - Ex: Yellow is the short leg, Green is the long leg

      - What does this mean?

        - Short leg (-) is goes to GND, Longer leg (+) goes to 3V

    - Take the wire attached to the shorter leg, and plug it into the negative column on the breadboard, in any row - do this for both LEDs

  - Connect LED 1

    - Take Alligator Clip 1, and clamp it to Pin 1 on Microbit

    - With other end of wire, plug it in to Row 1 on the breadboard

    - Take LED 1, and plug the remaining wire (which should be connected to the longer leg) and plug it into Row 1

  - Connect LED 2

    - Take Alligator Clip 2, and clamp it to Pin 2 on Microbit

    - With other end of wire, plug it in to Row 2 on the breadboard

    - Take LED 2, and plug its remaining wire (which should be connected to the longer leg) and plug it into Row 2

- Motor

  - Take 2 Pin/Pin wires, and 1 Pin/Hole wire and attach to the grouped wires coming from the motor

    - Keep track of which wire corresponds to each color of the grouped wires

    - The Pin/Hole wire should correspond to the RED wire coming from the motor

  - With the wire corresponding to the BLACK wire coming from the motor, plug it into the negative column on the breadboard

  - With the wire corresponding to the YELLOW wire coming from the motor, plug it into Row 3 on the breadboard

  - Take Alligator Clip 3, and clamp it on to Pin 0 on Microbit

  - With other end of the wire, plug it in to Row 3 on the breadboard

- GND

  - Take Alligator Clip 4, and clamp it to GND on Microbit

  - With other end of wire, plug in to negative column on breadboard in any row

- Batteries

  - Plug small battery back into microbit

  - Take the Pin/Hole wire attached to the red wire coming from the motor, and attach the F end to the Red wire coming from the big battery pack

  - Take the back wire coming from the big battery pack and plug it into the negative column on the breadboard in any row

  - Turn on the big battery pack